Three Mouse Functions-

1_) Mouse Movement – Move The Curser To any Direction on the screen

2) Left Click -  Used To Choose Files, Open Them

3) Right Click – used to Open File Opens (Rename, Detele, Move)

|  | FUNCTION | TRANSLATION |
| --- | --- | --- |
| 1) | Mouse Movement | Movement of the Head |
| 2) | Left Click | Single Blow on Mic |
| 3) | Right Click | Long Blow on mic(3Sec) |

ALGORITHM REQUIRED OPERATIONS:

- Addition

- Subtraction

- Multiplication

- Division

Function 1) MOVING CURSER – HEAD MOVEMENT

- OUR HEAD MOVEMENT DIRECTIONS

X AXIS:

Y- AXIS :

HOW DO WE MEASURE THIS MOVEMENT?

OUR SENSOR:

Accelerometer::

- STICKING THE SENSOR TO YOUR HEAD – AS THE HEAD MOVES THE SENSOR MOVES ACCORDINGLY.

- IT MEASURES THE CHANGE IN ITS MOTION ACROSS THE X AXIS AND Y AXIS.

- HEADMOVEMENTS IS MEASURED

X,Y,Z

(3 NUMBERS, EVERY 1 MILLISECONDS)

EXAMPLE OF MOVING OUR HEAD IN 5 SECONDS

1)MOVE OUR HEAD UPWARDS –

(X,0,Z) -> (x,20,Z)

2) MOVE HEAD TO RIGHT

(0,Y,Z) - >(30,Y,Z)

ANALYSING ACCELEROMETER DATA-

FORMAT:(three Numbers)

+X, + Y, + Z

1)MOVING OUR  HEAD UPWARDS: Y Changes WITH + SIGN

2) Moving HEAD DOWNWARDS: Y Changes WITH – SIGN

3) Moving Head To the RIGHT: X Changes WITH – SIGN

4) MOVING Head To LEFT: X Changes WITH + SIGN

ALGORITHM:

- Input Variables X = current X value of sensor ;Y=current Y value of sensor to the current sensor values.

Initialise prevX = 0; prevY=0;

- Process accelX = (X – prevX)/2;

Process accelY = (Y – prevY)/2;

- Processing the Scale: accelX = accelX * scalerX;

accelY = accelY * scalerY;

//AccelX and AccelY are the final Standard Mouse Coordinates

int x, y;

float Result;

void main(){

printf(“Enter x”);

scanf(“%d”, &x);

printf(“Enter y”);

scanf (“%d”, &y);//INPUTING

Result = x+y; //PROCESSING

Result = Result * 2;

Printf(“THE SUM IS %d”,Result);//OUTPUTING

}

Formulas:

Acceleration =

Change In Speed = Final Speed – Initial Speed

TIME

Angular Acceleration =

Change In Angle = Final Angle  - Initial Angle

Time(2)

Example: A object moves from 20km/sec to 40km/sec in 5 seconds. Then the change in speed or acceleration:

40 – 20     = 5

5

Example: A object changes angle from 60deg to 20deg in 5 seconds. Then the change in speed or acceleration:

60 – 20     = 8

5

THE PROCESSING:

1 Case: The Person Moves His Head To the Right in 2 Sec.

2 Case: The person Moves His to the Right in 5 Seconds.

Initialise Variables:

Int X – Current angle of our Head in X Axis

Int Y – Current Angle of Our Head In Y Axis

Int prevX – Last angle of our Head in X Axis

Int prevY – Last Angle of our head in Y Axis

Int accelX – acceleration in X Axis

Int accelY – Acceleration in Y Axis

Constant scalerX  = 10;

Constant ScalerY = 8;

Data From Sensor: (Sample)

0 , 0

0, +20 – (Moved Our Head Up)

-30, -10 – (Cross Downwards)

Data<0 -> Negative  (if(x<0) Else if(x==0) else(

Data>0 - > Positive

1Step: prevX = 0; prevY = 0;

x = 0;         y = 20;

accelX = (0 – 0)/2 = 0/2 = 0;

accelY = (20 – 0)/2 = 10; //accelY = (-20-0)/2 = -10

prevX = x = 0; prevY = y=+20;//This Gives Data For Next

accelY = 10 x ScalerY = 100;//Translation of Head Angles to Pixels

accelX = 0 x ScalerX = 0;

Coordinates – (0,100)

Case

step 2:

prevX = 0; prevY =+ 20;

x = -30; y = -10;

accelX = (-30-0)/2 = -15

accelY = (-10 – 20)/2 = -15

Understanding Computer Scales And Pixels:

1080 Pixels

720 Pixels

480 Pixels

- Our Computer Takes the coordinates of the curser with Pixels Unit;

- A Regular Computer Has Pixel Lengths upto 1000 to 800 Pixels

Data Of Sensor in a range of time:

0,0

0,0

0,0

0,0

0,0

0,0

0,10

0,10

0,10

0,0

0,-10

0,-10

5,0

4,0

4,0

3,0

Function 2 and 3: Left Click And Right Click

We use our mouth to make a blow on the mic to make a Left click and Right Click on the Screen.

Left Click: A Short Blow produces a left click

Right Click: A Long Blow Produces a Right Click.

How Do We Capture this Blow:

Sensor: AUDIO SENSOR with MIC

This sensor captures the sound in the mic and gives the loudness in scale of 0 – 1024 numbers.

The Number Increases with Loudness.

- Whether we are blowing or not.

Sol: If the Value is Above 600, we are blowing on mic.

If the value is belove 600, then we are not blowing.

Threshold Value = 600 This helps in differentiating.

- Whether the blow was long or short for left click or right click.

Sol: if the interval of change in values is more, or the set of values above 600 is more, the blow is longer.

If the set is between 3 and 5, then it is a short blow for LEFT CLICK.

If the set is between 5 and 10, the it is a long blow for Right Click.

0

0

0

0

620

605

650

0

0

0

0

620

605

650

640

630

670

650

0

0

0

0

0

THE PROJECT:

The parts That Provide Data as Input

- Accelerometer - Movement of our Head

MPU6050 Accelerometer + Gyroscope

- Mic Sensor – Duration and Blowing Status

RM008 SOUND SENSOR

Processing UNIT

- ARDUINO PRO MINI – uses Microcontroller Atmega328 -  FOR THE MOUSE UNIT

- ARDUINO MICRO – RECEIVER UNIT

Communication Part:

Bluetooth Module:

HC05 x 2 Units

One is connected to the mouse, another is connected receiving Side.

WireLess Mouse Has Two Parts:

- The Mouse

- The Receiving Pendrive That is connected to the computer. - Receiver

Parts Of The Project:

The Mouse Module:

- MPU6050 – Accelerometer

- RM008 – Sound Sensor

- Arduino Pro Mini – Microcontroller

- HC05 – Bluetooth Transmitter

- Power Source/Battery – 5V Output

- Over-The-Ear Headphone

The Receiver module:

- Arduino Micro – Microcontroller

- HC05 – Bluetooth Receiver

- USB CABLE

SOFTWARE: ARDUINO IDE

Language: C

Program Structure:

#include

//declartions

void setup() {

//RUNS AT START

}

void loop() {

//RUNS AFTER START IN LOOP

//RUNS 1000 Times per second

}

#include <stdio.h>

#include <mouseproject.h>

Int xglobal;

Float y;

Void function_name(int x, int y);

int main() {

printf("Hello, World!");

}

Void function_name(int x, int y){

Int z;

Z = x+y;

Printf(“%d”, z);

}

ARDUINO FUNCTIONS/MICROCONTROLLER FUNCTION:

- pinMode(2, OUTPUT); //setsup the mode of the given pin

- digitalWrite(2, HIGH);//SWITCH FOR PIN

- delay(1000);

- analogRead()

step1: led_2 should be on, led_3, Led_4 off

step2: wait .5 second

step3: led_3 on, led_2 led_3,off

step4: wait .5 seconds

step5: led_4 on, led2, led3 off

step6:wait 0.5 seconds

-All MICROCONTROLLERS HAVE A RUNNING FREQUENCY THAT OPERATE IN CYCLES

- 16 MHz

#include <stdio.h>

Int x;

Int main(){ //void setup

Printf

Scanf

file

While(1){//void loop

Printf(“hi”)

}

}

Project Electronics:

Preconcepts:

VCC – + ve INPUT POWER pin

5 volts

GND - -ve Input power

TXD-

RXD- COMMUNICATION LINES for Bluetooth

A0 – ANALOG PINS – Read Data From Mic Sensor

SDA

SCL – Communications lines for MPU6050 Accleretor – I2C

14,15 –

I2C Com. – Com bet Accerelometer n Micrrocontroller – (SCL,SDA,Int)

Serial Com – Bluetooth n Microcontroller(RXD, TXD)

#include <Wire.h>

#include "I2Cdev.h"

#include "MPU6050_6Axis_MotionApps20.h"

#include "MPU6050.h"

MPU6050 accelgyro(0X69);//I2C addreess for mpu6050 is 0X69

float X=0;

float Y=0;

float prevX=0;

float prevY=0;

float accelX=0;

float accelY=0;

float scalerX = 10;

float scalerY = 8;

int loopcount = 0;

int blowcount = 0;

void setup() {

Wire.begin();// put your setup code here, to run once:

Serial.begin(38400);

accelgyro.initialize();

}

void loop() {

accelgyro.getAcceleration(&X, &Y, &Z);

accelX = ((X - prevX)/2);

accelY = ((Y - prevX)/2);

prevX = X;

prevY = Y;

accelX = accelX * scalerX;

accelY = accelY * scalerY;

}

Click Processing Logic:

000…

630 800 700 605 640 670 900 601 740

0000

630 605 640 670

000000

Loopcount = 9

Loopcount = 4

Outputing:

Mouse Coordinates Format: “X , Y”

Mouse Click Signal Format:

“1” – LEFT Click – Blow

“2” – RIGHT Click – Long Blow

----------------------------------------------------

Click Output Variable: blowcount(int)

Coordinates Variable: mousecode(string): accelX + ”,” + accelY

Construction Flow:[METHODOLOGY]

- Ideating

- Model Designing and Planning

- Components Search and Market Selection

- Assign Microcontroller Pins on board

- Prepare The Circuit Diagram

- Start Making Connections, And Soldering

- Powerup And Test Connections And Communications

- Load The Our C Program

- Assemble DESIGN On HEADPHONE

- Glueing and Basing The Circuit Modules and Boards

- Powering Up And Prototype

- Test And Correction – Beta Testing

- Revise and Final Build

PowerBank – 50gms – Biggest

Connection Details:

A5------> SDA[mpu6050]

A4---- SCL[mpu6050]

Pin2----INT[mpu6050]

GND----AD0[mpu6050]

Rx ----Tx[Bluetooth]

Tx-----Rx[Bluetooth]

A0-----Mic Out[Rm00018]

A5,A4 – I2C Communication Pins- Address- {0X69}

SOLDERING WIRES:

Soldering Station

Soldering Flux

Soldering Lead

Single Core Multi Strand Insulated Wire

Strip the Insulation

Wind the strands

Apply solder

Insert in the board

Final solder

Connection Testing:

-Short Circuits

-Components OverHeat

-Power Leakage Or Oversupply

- Multimeter Terminal Test

- Test Programs to test Communication